MRP Simulator

## OBJECTIVE:

Develop an Excel-based MRP simulator backed by a PostgreSQL database to demonstrate practical knowledge of:

- Material Requirements Planning (MRP)

- Bill of Materials (BOM)

- Production Planning

- Inventory Management

- Purchasing

- SQL Database Design

- Excel-based planning and calculations

The PostgreSQL database will store all master and transactional data, while Excel will perform the MRP calculations and planning logic.

# PRODUCT STRUCTURE:

The project uses a 4-level BOM.

Level 0 – Finished Product (Centrifugal Water Pump)
Level 1 – Major Assemblies
Level 2 – Sub Assemblies / Purchased Components
Level 3 – Purchased Components

TABLES:

- suppliers

CREATE TABLE suppliers (

supplier_id INTEGER GENERATED ALWAYS AS IDENTITY PRIMARY KEY,

supplier_name VARCHAR(50) NOT NULL UNIQUE,

city VARCHAR(50) NOT NULL,

country VARCHAR(50) NOT NULL,

email VARCHAR(100) UNIQUE,

phone VARCHAR(20) UNIQUE,

is_active BOOLEAN NOT NULL DEFAULT TRUE

);

- items

CREATE TABLE items (

item_id INTEGER GENERATED ALWAYS AS IDENTITY PRIMARY KEY,

item_name VARCHAR(100) NOT NULL,

item_level INTEGER NOT NULL CHECK (item_level IN (0,1,2,3)),

item_category VARCHAR(50) NOT NULL CHECK (item_category IN ('Finished Product','Assembly','Component')),

procurement_type VARCHAR(50) NOT NULL CHECK (procurement_type IN ('Manufactured','Purchased')),

supplier_id INTEGER,

unit_of_measure VARCHAR(20) NOT NULL,

is_active BOOLEAN NOT NULL DEFAULT TRUE,

CONSTRAINT fk_supplier

FOREIGN KEY (supplier_id)

REFERENCES suppliers(supplier_id)

);

- bill_of_materials

CREATE TABLE bill_of_materials (

bom_id INTEGER GENERATED ALWAYS AS IDENTITY PRIMARY KEY,

parent_item_id INTEGER NOT NULL,

child_item_id INTEGER NOT NULL,

quantity_required DECIMAL(10,2) NOT NULL CHECK (quantity_required > 0),

sequence_no INTEGER,

CONSTRAINT fk_parent_item_id

FOREIGN KEY (parent_item_id)

REFERENCES items(item_id),

CONSTRAINT fk_child_item_id

FOREIGN KEY (child_item_id)

REFERENCES items(item_id),

CONSTRAINT uq_parent_child_relationship

UNIQUE (parent_item_id, child_item_id),

CONSTRAINT different_parent_child

CHECK (parent_item_id <> child_item_id)

);

- inventory

CREATE TABLE inventory (

item_id INTEGER PRIMARY KEY,

quantity_on_hand DECIMAL(10,2) NOT NULL DEFAULT 0 CHECK (quantity_on_hand >= 0),

allocated_quantity DECIMAL(10,2) NOT NULL DEFAULT 0 CHECK (allocated_quantity >= 0),

available_quantity DECIMAL(10,2) GENERATED ALWAYS AS (quantity_on_hand - allocated_quantity) STORED,

last_updated TIMESTAMP NOT NULL DEFAULT CURRENT_TIMESTAMP,

CONSTRAINT fk_inventory_item

FOREIGN KEY (ITEM_ID)

REFERENCES items(item_id),

CONSTRAINT allocated_not_exceed_on_hand

CHECK (allocated_quantity <= quantity_on_hand)

);

- customer_orders

CREATE TABLE customer_orders (

customer_order_id INTEGER GENERATED ALWAYS AS IDENTITY PRIMARY KEY,

customer_name VARCHAR(50) NOT NULL,

item_id INTEGER NOT NULL,

order_quantity DECIMAL(10,2) NOT NULL CHECK (order_quantity > 0),

order_date DATE NOT NULL,

due_date DATE NOT NULL CHECK (due_date >= order_date),

order_status VARCHAR(20) NOT NULL CHECK (order_status IN ('Pending','Released','Completed','Cancelled')),

CONSTRAINT fk_customer_order_item

FOREIGN KEY (item_id)

REFERENCES items(item_id)

);

- production_orders

CREATE TABLE production_orders (

production_order_id INTEGER GENERATED ALWAYS AS IDENTITY PRIMARY KEY,

customer_order_id INTEGER NOT NULL,

item_id INTEGER NOT NULL,

production_quantity DECIMAL(10,2) NOT NULL CHECK (production_quanityt > 0),

start_date DATE NOT NULL,

completion_date DATE NOT NULL CHECK (completion_date >= start_date),

production_status VARCHAR(20) NOT NULL CHECK (production_status IN ('Pending','Released','Completed','Cancelled')),

CONSTRAINT fk_production_customer_order

FOREIGN KEY (customer_order_id)

REFERENCES customer_orders(customer_order_id),

CONSTRAINT fk_production_item

FOREIGN KEY (item_id)

REFERENCES items(item_id)

);

- purchase_orders

CREATE TABLE purchase_orders (

purchase_order_id INTEGER GENERATED ALWAYS AS IDENTITY PRIMARY KEY,

production_order_id INTEGER NOT NULL,

supplier_id INTEGER NOT NULL,

item_id INTEGER NOT NULL,

purchase_quantity DECIMAL(10,2) NOT NULL CHECK (purchase_quantity > 0),

order_date DATE NOT NULL

delivery_date DATE NOT NULL CHECK (delivery_date >= order_date),

purchase_status VARCHAR(20) NOT NULL CHECK (purchase_status IN ('Pending','Released','Completed','Cancelled'))

CONSTRAINT fk_purchase_production_order

FOREIGN KEY (production_order_id)

REFERENCES production_orders(production_order_id),

CONSTRAINT fk_purchase_supplier

FOREIGN KEY (supplier_id)

REFERENCES suppliers(supplier_id),

CONSTRAINT fk_purchase_item

FOREIGN KEY (item_id)

REFERENCES items(item_id)

);

# DATABASE RELATIONSHIPS:

| Parent Table | Child Table | Relationship |
| --- | --- | --- |
| suppliers | items | One supplier → Many purchased items |
| items | bill_of_materials | One item can be parent or child in many BOM records |
| items | inventory | One item → One inventory record |
| items | customer_orders | One item → Many customer orders |
| customer_orders | production_orders | One customer order → Many production orders |
| production_orders | purchase_orders | One production order → Many purchase orders |
| items | production_orders | One item → Many production orders |
| items | purchase_orders | One item → Many purchase orders |
| suppliers | purchase_orders | One supplier → Many purchase orders |

# LOCKED BUSINESS RULES:

- One finished product (Industrial Centrifugal Water Pump)

- Four-level BOM (Levels 0–3)

- Purchased items may exist at both Level 2 and Level 3

- One supplier supplies many purchased items

- One purchased item has exactly one supplier

- One inventory record per item

- Available quantity is a generated column

- Duplicate parent-child BOM relationships are not allowed

- Duplicate supplier names are not allowed

- One customer order may generate multiple production orders

- One production order may generate multiple purchase orders

INSERTING IN TABLES:

- suppliers

INSERT INTO suppliers (supplier_name, city, country, email, phone)

VALUES

('SKF India Ltd.', 'Pune', 'India', 'contact@skfindia.com', '+91-20-40010001'),

('Trelleborg Sealing Solutions', 'Bengaluru', 'India', 'sales@trelleborg.com', '+91-80-40010002'),

('L&T Valves', 'Chennai', 'India', 'info@ltvalves.com', '+91-44-40010003'),

('Tata Steel Processing', 'Jamshedpur', 'India', 'support@tatasteel.com', '+91-657-4001004'),

('Hindalco Industries', 'Mumbai', 'India', 'sales@hindalco.com', '+91-22-40010005'),

('RR Kabel', 'Vadodara', 'India', 'contact@rrkabel.com', '+91-265-40010006'),

('Saint-Gobain India', 'Chennai', 'India', 'info@saint-gobain.com', '+91-44-40010007'),

('Fasteners India Pvt. Ltd.', 'Ludhiana', 'India', 'sales@fastenersindia.com', '+91-161-40010008'),

('Packwell Packaging', 'Noida', 'India', 'info@packwell.com', '+91-120-40010009'),

('C&S Electric', 'Delhi', 'India', 'support@cselectric.com', '+91-11-40010010');

- items

INSERT INTO items

(item_name, item_level, item_category, procurement_type, supplier_id, unit_of_measure)

VALUES

-- Level 0

('Industrial Centrifugal Water Pump', 0, 'Finished Product', 'Manufactured', NULL, 'EA'),

-- Level 1

('Pump Assembly', 1, 'Assembly', 'Manufactured', NULL, 'EA'),

('Electric Motor Assembly', 1, 'Assembly', 'Manufactured', NULL, 'EA'),

('Base Frame Assembly', 1, 'Assembly', 'Manufactured', NULL, 'EA'),

('Packaging Assembly', 1, 'Assembly', 'Manufactured', NULL, 'EA'),

-- Level 2 - Pump Assembly

('Casing Assembly', 2, 'Assembly', 'Manufactured', NULL, 'EA'),

('Rotating Assembly', 2, 'Assembly', 'Manufactured', NULL, 'EA'),

('Mechanical Seal Assembly', 2, 'Assembly', 'Manufactured', NULL, 'EA'),

('Suction Flange', 2, 'Component', 'Purchased', 3, 'EA'),

('Discharge Flange', 2, 'Component', 'Purchased', 3, 'EA'),

-- Level 2 - Electric Motor Assembly

('Motor Housing Assembly', 2, 'Assembly', 'Manufactured', NULL, 'EA'),

('Rotor Assembly', 2, 'Assembly', 'Manufactured', NULL, 'EA'),

('Stator Assembly', 2, 'Assembly', 'Manufactured', NULL, 'EA'),

('Cooling Fan Assembly', 2, 'Assembly', 'Manufactured', NULL, 'EA'),

('Terminal Box Assembly', 2, 'Assembly', 'Manufactured', NULL, 'EA'),

-- Level 2 - Base Frame Assembly

('Base Plate', 2, 'Component', 'Purchased', 4, 'EA'),

('Mounting Feet', 2, 'Component', 'Purchased', 4, 'EA'),

('Coupling Guard', 2, 'Component', 'Purchased', 4, 'EA'),

('Anti Vibration Pads', 2, 'Component', 'Purchased', 2, 'EA'),

('Anchor Bolts', 2, 'Component', 'Purchased', 8, 'EA'),

('Anchor Nuts', 2, 'Component', 'Purchased', 8, 'EA'),

('Flat Washers', 2, 'Component', 'Purchased', 8, 'EA'),

-- Level 2 - Packaging Assembly

('Carton Box', 2, 'Component', 'Purchased', 9, 'EA'),

('Foam Inserts', 2, 'Component', 'Purchased', 9, 'EA'),

('Plastic Bag', 2, 'Component', 'Purchased', 9, 'EA'),

('User Manual', 2, 'Component', 'Purchased', 9, 'EA'),

('Warranty Card', 2, 'Component', 'Purchased', 9, 'EA'),

('Product Label', 2, 'Component', 'Purchased', 9, 'EA'),

-- Level 3 - Casing Assembly

('Pump Casing', 3, 'Component', 'Purchased', 4, 'EA'),

('Casing Cover', 3, 'Component', 'Purchased', 4, 'EA'),

('Wear Ring', 3, 'Component', 'Purchased', 1, 'EA'),

('Casing Gasket', 3, 'Component', 'Purchased', 7, 'EA'),

('Drain Plug', 3, 'Component', 'Purchased', 3, 'EA'),

('Hex Bolt M12', 3, 'Component', 'Purchased', 8, 'EA'),

('Hex Nut M12', 3, 'Component', 'Purchased', 8, 'EA'),

('Flat Washer', 3, 'Component', 'Purchased', 8, 'EA'),

('Spring Washer', 3, 'Component', 'Purchased', 8, 'EA'),

-- Level 3 - Rotating Assembly

('Pump Shaft', 3, 'Component', 'Purchased', 4, 'EA'),

('Impeller', 3, 'Component', 'Purchased', 4, 'EA'),

('Front Bearing', 3, 'Component', 'Purchased', 1, 'EA'),

('Rear Bearing', 3, 'Component', 'Purchased', 1, 'EA'),

('Bearing Housing', 3, 'Component', 'Purchased', 4, 'EA'),

('Shaft Sleeve', 3, 'Component', 'Purchased', 4, 'EA'),

('Shaft Key', 3, 'Component', 'Purchased', 8, 'EA'),

('Bearing Lock Nut', 3, 'Component', 'Purchased', 8, 'EA'),

('Retaining Ring', 3, 'Component', 'Purchased', 8, 'EA'),

('Spacer Ring', 3, 'Component', 'Purchased', 4, 'EA'),

('Circlip', 3, 'Component', 'Purchased', 8, 'EA'),

-- Level 3 - Mechanical Seal Assembly

('Rotating Seal Ring', 3, 'Component', 'Purchased', 2, 'EA'),

('Stationary Seal Ring', 3, 'Component', 'Purchased', 2, 'EA'),

('Spring', 3, 'Component', 'Purchased', 8, 'EA'),

('O-Ring', 3, 'Component', 'Purchased', 2, 'EA'),

('Seal Retainer', 3, 'Component', 'Purchased', 2, 'EA'),

('Drive Pin', 3, 'Component', 'Purchased', 8, 'EA'),

('Retaining Clip', 3, 'Component', 'Purchased', 8, 'EA'),

-- Level 3 - Motor Housing Assembly

('Motor Housing', 3, 'Component', 'Purchased', 4, 'EA'),

('Fan Cover', 3, 'Component', 'Purchased', 4, 'EA'),

('Nameplate', 3, 'Component', 'Purchased', 9, 'EA'),

('Cable Gland', 3, 'Component', 'Purchased', 10, 'EA'),

('Mounting Bolt', 3, 'Component', 'Purchased', 8, 'EA'),

('Mounting Nut', 3, 'Component', 'Purchased', 8, 'EA'),

('Lock Washer', 3, 'Component', 'Purchased', 8, 'EA'),

-- Level 3 - Rotor Assembly

('Rotor Shaft', 3, 'Component', 'Purchased', 4, 'EA'),

('Rotor Core', 3, 'Component', 'Purchased', 4, 'EA'),

('Copper Bars', 3, 'Component', 'Purchased', 6, 'EA'),

('End Rings', 3, 'Component', 'Purchased', 5, 'EA'),

('Rotor Retaining Ring', 3, 'Component', 'Purchased', 8, 'EA'),

('Balancing Weight', 3, 'Component', 'Purchased', 5, 'EA'),

-- Level 3 - Stator Assembly

('Stator Core', 3, 'Component', 'Purchased', 4, 'EA'),

('Copper Windings', 3, 'Component', 'Purchased', 6, 'EA'),

('Insulation Paper', 3, 'Component', 'Purchased', 7, 'EA'),

('Slot Wedges', 3, 'Component', 'Purchased', 7, 'EA'),

('Terminal Leads', 3, 'Component', 'Purchased', 10, 'EA'),

('Insulating Sleeves', 3, 'Component', 'Purchased', 10, 'EA'),

('Cable Ties', 3, 'Component', 'Purchased', 10, 'EA'),

('Varnish Pack', 3, 'Component', 'Purchased', 7, 'EA'),

-- Level 3 - Cooling Fan Assembly

('Cooling Fan', 3, 'Component', 'Purchased', 5, 'EA'),

('Fan Hub', 3, 'Component', 'Purchased', 5, 'EA'),

('Set Screw', 3, 'Component', 'Purchased', 8, 'EA'),

-- Level 3 - Terminal Box Assembly

('Terminal Box', 3, 'Component', 'Purchased', 10, 'EA'),

('Terminal Block', 3, 'Component', 'Purchased', 10, 'EA'),

('Terminal Cover', 3, 'Component', 'Purchased', 10, 'EA'),

('Terminal Screw', 3, 'Component', 'Purchased', 8, 'EA'),

('Terminal Nut', 3, 'Component', 'Purchased', 8, 'EA'),

('Cable Clamp', 3, 'Component', 'Purchased', 10, 'EA');

item_id, item_name OUTPUT:

"item_id"	"item_name"

1		"Industrial Centrifugal Water Pump"

2		"Pump Assembly"

3		"Electric Motor Assembly"

4		"Base Frame Assembly"

5		"Packaging Assembly"

6		"Casing Assembly"

7		"Rotating Assembly"

8		"Mechanical Seal Assembly"

9		"Suction Flange"

10		"Discharge Flange"

11		"Motor Housing Assembly"

12		"Rotor Assembly"

13		"Stator Assembly"

14		"Cooling Fan Assembly"

15		"Terminal Box Assembly"

16		"Base Plate"

17		"Mounting Feet"

18		"Coupling Guard"

19		"Anti Vibration Pads"

20		"Anchor Bolts"

21		"Anchor Nuts"

22		"Flat Washers"

23		"Carton Box"

24		"Foam Inserts"

25		"Plastic Bag"

26		"User Manual"

27		"Warranty Card"

28		"Product Label"

29		"Pump Casing"

30		"Casing Cover"

31		"Wear Ring"

32		"Casing Gasket"

33		"Drain Plug"

34		"Hex Bolt M12"

35		"Hex Nut M12"

36		"Flat Washer"

37		"Spring Washer"

38		"Pump Shaft"

39		"Impeller"

40		"Front Bearing"

41		"Rear Bearing"

42		"Bearing Housing"

43		"Shaft Sleeve"

44		"Shaft Key"

45		"Bearing Lock Nut"

46		"Retaining Ring"

47		"Spacer Ring"

48		"Circlip"

49		"Rotating Seal Ring"

50		"Stationary Seal Ring"

51		"Spring"

52		"O-Ring"

53		"Seal Retainer"

54		"Drive Pin"

55		"Retaining Clip"

56		"Motor Housing"

57		"Fan Cover"

58		"Nameplate"

59		"Cable Gland"

60		"Mounting Bolt"

61		"Mounting Nut"

62		"Lock Washer"

63		"Rotor Shaft"

64		"Rotor Core"

65		"Copper Bars"

66		"End Rings"

67		"Rotor Retaining Ring"

68		"Balancing Weight"

69		"Stator Core"

70		"Copper Windings"

71		"Insulation Paper"

72		"Slot Wedges"

73		"Terminal Leads"

74		"Insulating Sleeves"

75		"Cable Ties"

76		"Varnish Pack"

77		"Cooling Fan"

78		"Fan Hub"

79		"Set Screw"

80		"Terminal Box"

81		"Terminal Block"

82		"Terminal Cover"

83		"Terminal Screw"

84		"Terminal Nut"

85		"Cable Clamp"

- bill_of_materials

INSERT INTO bill_of_materials

(parent_item_id, child_item_id, quantity_required, sequence_no)

VALUES

-- Level 0 → Level 1

(1,2,1,1),

(1,3,1,2),

(1,4,1,3),

(1,5,1,4),

-- Pump Assembly

(2,6,1,1),

(2,7,1,2),

(2,8,1,3),

(2,9,1,4),

(2,10,1,5),

-- Electric Motor Assembly

(3,11,1,1),

(3,12,1,2),

(3,13,1,3),

(3,14,1,4),

(3,15,1,5),

-- Base Frame Assembly

(4,16,1,1),

(4,17,2,2),

(4,18,1,3),

(4,19,4,4),

(4,20,4,5),

(4,21,4,6),

(4,22,4,7),

-- Packaging Assembly

(5,23,1,1),

(5,24,2,2),

(5,25,1,3),

(5,26,1,4),

(5,27,1,5),

(5,28,2,6),

-- Casing Assembly

(6,29,1,1),

(6,30,1,2),

(6,31,2,3),

(6,32,1,4),

(6,33,2,5),

(6,34,12,6),

(6,35,12,7),

(6,36,12,8),

(6,37,12,9),

-- Rotating Assembly

(7,38,1,1),

(7,39,1,2),

(7,40,1,3),

(7,41,1,4),

(7,42,1,5),

(7,43,1,6),

(7,44,1,7),

(7,45,2,8),

(7,46,4,9),

(7,47,2,10),

(7,48,2,11),

-- Mechanical Seal Assembly

(8,49,1,1),

(8,50,1,2),

(8,51,1,3),

(8,52,4,4),

(8,53,1,5),

(8,54,2,6),

(8,55,2,7),

-- Motor Housing Assembly

(11,56,1,1),

(11,57,1,2),

(11,58,1,3),

(11,59,2,4),

(11,60,8,5),

(11,61,8,6),

(11,36,8,7),

(11,62,8,8),

-- Rotor Assembly

(12,63,1,1),

(12,64,1,2),

(12,65,28,3),

(12,66,2,4),

(12,67,2,5),

(12,68,4,6),

(12,62,2,7),

-- Stator Assembly

(13,69,1,1),

(13,70,6,2),

(13,71,24,3),

(13,72,24,4),

(13,73,3,5),

(13,74,12,6),

(13,75,12,7),

(13,76,1,8),

-- Cooling Fan Assembly

(14,77,1,1),

(14,78,1,2),

(14,79,2,3),

(14,62,2,4),

-- Terminal Box Assembly

(15,80,1,1),

(15,81,1,2),

(15,82,1,3),

(15,83,6,4),

(15,84,6,5),

(15,36,6,6),

(15,85,2,7);

- inventory

INSERT INTO inventory (item_id, quantity_on_hand, allocated_quantity)

VALUES

-- Level 0

(1,0,0),

-- Level 1 Assemblies

(2,0,0),

(3,0,0),

(4,0,0),

(5,0,0),

-- Level 2

(6,1,0),

(7,0,0),

(8,2,0),

(9,5,1),

(10,4,1),

(11,1,0),

(12,0,0),

(13,1,0),

(14,2,0),

(15,1,0),

(16,5,1),

(17,6,1),

(18,4,1),

(19,5,1),

(20,10,2),

(21,10,2),

(22,10,2),

(23,5,1),

(24,6,1),

(25,5,1),

(26,8,0),

(27,8,0),

(28,8,0),

-- Casing Assembly Components

(29,3,0),

(30,2,0),

(31,4,1),

(32,6,1),

(33,5,1),

(34,10,2),

(35,10,2),

(36,10,2),

(37,10,2),

-- Rotating Assembly Components

(38,2,0),

(39,3,0),

(40,4,1),

(41,2,0),

(42,2,0),

(43,4,1),

(44,6,1),

(45,5,1),

(46,6,1),

(47,5,1),

(48,6,1),

-- Mechanical Seal Components

(49,4,1),

(50,4,1),

(51,6,1),

(52,8,2),

(53,4,1),

(54,5,1),

(55,5,1),

-- Motor Housing Components

(56,2,0),

(57,3,0),

(58,6,1),

(59,5,1),

(60,8,2),

(61,8,2),

(62,8,2),

-- Rotor Components

(63,2,0),

(64,2,0),

(65,8,2),

(66,5,1),

(67,5,1),

(68,6,1),

-- Stator Components

(69,2,0),

(70,8,2),

(71,10,2),

(72,8,2),

(73,6,1),

(74,5,1),

(75,6,1),

(76,4,1),

-- Cooling Fan Components

(77,4,1),

(78,4,1),

(79,6,1),

-- Terminal Box Components

(80,4,1),

(81,5,1),

(82,5,1),

(83,8,2),

(84,8,2),

(85,5,1);

- customer_orders

INSERT INTO customer_orders

(customer_name, item_id, order_quantity, order_date, due_date, order_status)

VALUES

('ABC Industries', 1, 5, '2026-08-01', '2026-08-10', 'Pending'),

('Delta Manufacturing', 1, 3, '2026-08-02', '2026-08-12', 'Released'),

('HydroTech Solutions', 1, 8, '2026-08-03', '2026-08-15', 'Pending'),

('Prime Engineering', 1, 2, '2026-08-05', '2026-08-13', 'Completed'),

('Apex Pumps Ltd.', 1, 6, '2026-08-06', '2026-08-18', 'Pending'),

('Global Water Systems', 1, 4, '2026-08-08', '2026-08-17', 'Released'),

('Vertex Engineering', 1, 7, '2026-08-09', '2026-08-20', 'Pending'),

('BlueRiver Industries', 1, 3, '2026-08-10', '2026-08-19', 'Cancelled'),

('Metro Process Equipments', 1, 9, '2026-08-11', '2026-08-25', 'Pending'),

('Nova Industrial Works', 1, 4, '2026-08-12', '2026-08-22', 'Released');

customer_order_id, customer_name OUTPUT:

customer_order_id  	customer_name

1			"ABC Industries"

2			"Delta Manufacturing"

3			"HydroTech Solutions"

4			"Prime Engineering"

5			"Apex Pumps Ltd."

6			"Global Water Systems"

7			"Vertex Engineering"

8			"BlueRiver Industries"

9			"Metro Process Equipments"

10			"Nova Industrial Works"